# Dryve D1 - Python / Modbus TCP Gateway Guide

Disclaimer

Sample program for igus dryve motor controls

Thank you very much for choosing our motor controls. In the following you will find some important information about the use of our sample programs. For all sample programs the appropriate dryve D1 configuration (.txt) must be loaded. This is included in the download package.

If the sample program contains ready-made functions or function blocks, please take the function description from the block itself (variable declaration).

Please always use the latest motor control firmware from the igus dryve website.

For any questions regarding the commissioning of the D1 motor control please contact us: DE-dryve@igus.net

If you have any questions about programming with Siemens TIA Portal please contact Siemens Support.

The sample program and guides are free of charge and may be used freely. Company igus does not give any support for third-party products or third-party software. Also no responsibility/liability is assumed for the program, guides.

Purpose

This guide fills in the practical link between the byte arrays used in the Python samples and the Modbus TCP Gateway / CANopen objects described in the D1 manual.

# 1. Overview

The Python sample programs are not using a separate command set from the D1 manual. The arrays in the Python code are the Ethernet telegrams sent to the D1 through its Modbus TCP Gateway.

Python function  →  Modbus TCP Gateway telegram  →  CANopen SDO object  →  D1 action

The D1 manual explains this in two places. Section 5.7 explains how the telegram is packaged and transferred over Ethernet. The motion-control/CANopen sections explain what the object being read or written actually means.

This is why looking only at section 5.7 can feel incomplete: it explains the transport format, while the meaning of objects such as 6040h, 6041h, 6060h and 607Ah is defined elsewhere in the manual.

# 2. Array Variable Length

The sample code contains arrays of different lengths; they are not always 21 values. A read request ends after byte 18 (19 bytes total). A write request adds 1–4 data bytes, so it is 20–23 bytes total. For example, the Controlword write below is 21 bytes because object 6040h is two bytes long.

shutdown = [0,0,0,0,0,15,0,43,13,1,0,0,96,64,0,0,0,0,2,6,0]

# 3. Decoding a Telegram

Using the Sample 9 / Sample 12 Shutdown telegram as the worked example:

| Byte(s) | Value | Meaning |
| --- | --- | --- |
| 0–1 | 0, 0 | Transaction Identifier. The sample uses 0 here; it can also be used to associate a response with a request. |
| 2–3 | 0, 0 | Protocol Identifier = Modbus. |
| 4 | 0 | Unused, but must be sent. |
| 5 | 15 | Number of bytes following byte 5. A two-byte SDO write gives 15. |
| 6 | 0 | Unit Identifier; unused here but must be sent. |
| 7 | 43 (2Bh) | Function code for Modbus TCP Gateway (CANopen). |
| 8 | 13 (0Dh) | MEI type for the CANopen gateway. |
| 9 | 1 | Protocol control: 0 = read, 1 = write. |
| 10–11 | 0, 0 | Reserved / Node ID; unused in this implementation. |
| 12–13 | 96, 64 | Object Index. 96 decimal = 60h and 64 decimal = 40h, therefore object 6040h. |
| 14 | 0 | Sub-index 0. |
| 15–17 | 0, 0, 0 | Unused fields which must still be sent. |
| 18 | 2 | Byte count: object 6040h is two bytes long. |
| 19–20 | 6, 0 | Data field, little-endian. This represents 0006h. |

The result is therefore: write the value 0006h to object 6040h. Object 6040h is the Controlword, and 0006h is the Shutdown command used by the D1 state machine.

6040h (Controlword)  +  0006h  =  Shutdown

# 4. Conversion of 96, 64 to 6040h

The Python arrays are written as decimal byte values, while the D1 object dictionary in the manual uses hexadecimal object numbers. Convert each object-index byte individually:

96 decimal = 60h
64 decimal = 40h

[96, 64]  →  60 40h  →  object 6040h

The same method works throughout the samples. For example, [96, 122] becomes [60h, 7Ah], which is object 607Ah (Target Position).

# 5. Sample Program Frequently Used Objects

| Object | Purpose in the sample |
| --- | --- |
| 6040h | Controlword – commands the D1 state machine and starts movements. |
| 6041h | Statusword – read back to determine controller/movement status. |
| 6060h | Modes of Operation – selects the requested operating mode. |
| 6061h | Modes of Operation Display – confirms the mode actually set. |
| 607Ah | Target Position. |
| 6081h | Profile Velocity. |
| 6083h | Profile Acceleration. |
| 6092h | Feed Constant – used by Sample 12 when converting engineering units. |
| 6099h | Homing Speeds. |
| 609Ah | Homing Acceleration. |
| 60A8h | SI Unit Position – used by Sample 12 to establish the unit conversion factor. |

# 6. Sample 12 functions in relation to the D1 Dryve

Sample 12 already comments the main objects used by each function. The table below is the practical bridge between the Python function names and the D1 objects.

| Python function | What it does at D1 level |
| --- | --- |
| sendCommand(data) | Sends the prepared telegram over the TCP socket and receives the D1 response. |
| to_four_byte(x) | Converts an integer into the four data bytes required for a 4-byte object value. |
| set_shutdn() | Writes 0006h to Controlword 6040h, then reads Statusword 6041h until the expected state is returned. |
| set_swon() | Writes 0007h to Controlword 6040h, then monitors Statusword 6041h. |
| set_op_en() | Writes 000Fh to Controlword 6040h, then monitors Statusword 6041h. |
| initialize() | Resets state/error handling and runs Shutdown → Switch On → Enable Operation. |
| set_mode(mode) | Writes the requested mode to 6060h and reads 6061h until that mode is confirmed. |
| homing(...) | Selects Homing mode (6), writes homing speed 6099h:01 and acceleration 609Ah, then starts the movement through 6040h and monitors 6041h. |
| profile_pos_mode(...) | Selects Profile Position mode (1), writes 6081h velocity, 6083h acceleration and 607Ah target position, then starts the movement via 6040h and monitors 6041h. |

# 7. Profile Position movement

In Sample 12, a profile-position command can be followed in this order:

1. set_mode(1) writes mode 1 to 6060h (Profile Position Mode) and checks 6061h.

2. The requested velocity is converted into the D1 units and written to 6081h.

3. The requested acceleration is converted and written to 6083h.

4. The requested position is converted and written to 607Ah.

5. The Enable Operation Controlword is sent so bit 4 is low before the next start edge.

6. 001Fh is written to Controlword 6040h; this provides the high flank on bit 4 used by the sample to start the movement.

7. Statusword 6041h is repeatedly read until the program sees an accepted completion/status condition.

# 8. Reading / Writing

Byte 9 is the quickest way to see whether a telegram is requesting information or changing a D1 object:

| Byte 9 | Operation | What follows |
| --- | --- | --- |
| 0 | Read | The request identifies the object and expected byte count. The returned telegram carries the object's data field. |
| 1 | Write | The request identifies the object and includes 1–4 data bytes to be written. |

The gateway does not automatically stream Statusword, position or other values. The manual states that a read telegram must be sent whenever the master requires information. This is why the Python code repeatedly calls the Statusword read while it waits for a state or movement to complete.

# 9. Endianness

The object index in bytes 12–13 is presented in the same order used in the manual examples (for example 60h, 40h for 6040h). The data field in bytes 19–22 is little-endian.

Two-byte value 0006h  →  bytes 19–20 = [6, 0]
Four-byte value 6000 decimal  →  bytes 19–22 = [112, 23, 0, 0]

For four-byte motion values, Sample 12 uses to_four_byte() to create the required data bytes automatically.

# 10. Start-up sequence for the supplied sample

The sample package includes its own one-page instruction. The important point is that the supplied configuration is not merely a DHCP switch. The sample configuration establishes a known starting configuration and must then be adapted to the actual axis.

1. Use the appropriate/latest D1 firmware for the sample and controller.

2. Load the configuration file supplied with the sample package.

3. With Modbus TCP Gateway / Enable DI7 disabled while configuring, adapt the D1 to the actual application: motor, gearbox, homing/reference method, limit switches, stroke, feed, speed, acceleration/deceleration, following error and other relevant settings.

4. Activate Modbus TCP Gateway and the required DI7 enable arrangement.

5. Restart the D1.

6. Confirm the controller's IP address and make sure it matches the IP address used in the Python program.

7. Do not command movement values above the Motion Limits configured in the D1.

8. Run the Python sample and begin with a simple state/communication check before adapting the movement sequence.

Important clarification on DHCP: the supplied Sample 9 and Sample 12 configuration files have DHCP disabled, and the Sample 12 instruction states that after restart the web interface is reached at 169.254.0.1. Therefore the reason for loading the configuration file is not simply to switch DHCP on. The configuration files also contain motor, actuator, communication, drive-profile and other settings. Those application-specific values must still be checked and adjusted for the real hardware.

# 11. Code Work- Through Guide

1. Find the Python function you want to understand.
2. Find the telegram(s) it sends.
3. Check byte 9: read (0) or write (1).
4. Read bytes 12–13 and convert decimal bytes to hex.
5. Search that object number in the D1 manual.
6. Check byte 14 for the sub-index.
7. Check byte 18 for the data length.
8. For a write, decode bytes 19–22 as the little-endian value.
9. For a read, inspect the same data area in the D1 response.
10. Use the object's manual definition to understand the resulting D1 behaviour.

# 12. Where to look in the D1 V3.3 manual

| Manual section | Use it for |
| --- | --- |
| 5.7.1 Motion Control via Modbus TCP as Gateway | Explains that Modbus TCP Gateway uses the D1's CANopen motion-control behaviour and SDO communication. |
| 5.7.2 Necessary User Interface Settings | Motor, communication and Drive Profile prerequisites. |
| 5.7.3 Conversion Decimal into Double Word Decimal | Examples of converting values into data bytes. |
| 5.7.4 Communication Verification | A practical check using 6041h and 607Ah. |
| 5.7.5 Byte Assignment Modbus TCP Gateway Telegram | Meaning of bytes 0–22. |
| 5.7.6 Configuration Read/Write Telegram | Which bytes are sent for 1–4 byte reads/writes. |
| 5.7.7 RX/TX Telegram Example | Worked State Machine, Homing and Profile Position telegrams. |
| 5.6.6–5.6.10 / detailed motion-control objects | State Machine, Statusword, Controlword, operating modes and the meaning of individual objects. |

# 13. Bottom line

The missing link is that section 5.7 describes the envelope used to transport CANopen SDO object reads/writes over Ethernet, while the motion-control object sections describe the meaning of those objects. Sample Programs 9 and 12 then assemble those pieces into Python functions.

Python function  →  object read/write telegram  →  CANopen object definition  →  D1 behaviour

Once the object index in bytes 12–13 is identified, the arrays become much easier to follow. For development, it is usually more useful to trace the object being accessed than to treat the entire array as an opaque command.

# Sources

dryve D1 Operating Manual V3.3;
Sample Program 9 (D1-1-PC-ModbusTCP(GW)-9-V1.1);
Sample Program 12 (D1-2-PC-ModbusTCP(GW)-12-V1.3);
Instruction /configuration files supplied in the above sample packages.

The guide intentionally does not assume that the supplied configuration is suitable for a particular machine. Motor, axis, homing, stroke, feed, limits and other application-specific parameters must be set for the actual system.